ECTE202 summary

- Electrical systems: combination of circuits and non-electrical parts performing an integrated task

- Best approach is top down

Dependant sources

- A pair of terminals might be called a port

- Node voltage

  - If there is a branch with only a voltage source you can create a super node. eg. Vm and Vm + 5

  - With dependant sources simply add their equations to the KCL equations

- Mesh Current

  - only works on planar circuits

  - If there is current source in branch we can form an equation relating the current loops

- Only need node voltage analysis in ECTE202

- Voltage source convert to current source vice versa

- Superpositioning

  - dependant sources

    - Do as per normal, treat dependant sources like resistors

- Thevinin/Norton

  - Find SC current and OC voltage, then divide to find resistance

  - in circuit without source need to replace cap/ind with 1v source or 1A source.

- Max power transfer is at Rth

- opamps take weak signals and boosts them to usable levels

  - they are 1 or more input ports and one output port. The output is a mathematical operation on the inputs.

  - The output is the (non-inverting input – inverting input) times gain

  - Assume perfect characteristics for course

  - Voltage is same at input terminals

  - no current enters opamp

  - gain of opamp is hard to control so feed back is used to provide stable gain

  - We only consider negative feed back circuits

  - V_o = A(v+ - v-)

- Storage elements

  - v = H di/dt

  - I = C dv/dt

  - dE = v C dv

    - E = ½ CV2

  - dE = i L dt

    - E = ½ Li2

  - capacitors in series act as resistors in parallel

  - series inductors act as series resistors

- Natural response

  - when capacitor isn’t charging KVL gives Vc + Ri = 0, but i = C dv/dt

  - source free equation!!!!

  - VC = Aest

  - di/dt can be replaced by s when t = 0

  - VC = Aet/tau

  - Tau = RC

  - Tau = L/R

  - Tau = -1/s

  - at t = 0

    - power = V2/R

  - CR = NR + FR, find A last

  - for pulse consider 2 steps

  - For multiple sources use super positioning

  - NR is still from source free equation

  - For non-constant sources the output follows the same form as the input

  - to solve equate coefficients

- non-constant source

  - v = Lsi

    - v/i = Ls

  - i = Csv

    - v/i = 1/cs

  - if NR and FR have same time constant need to assume Btest

  - FR has same form as input

- 2nd order

  - obtain characteristic equation using the s-operator

  - 3 kinds of natural response

    - Aeat + Beb

    - Aeat + Bte-at

    - (Asinbt + B cosbt)eat

  - to find forced response

    - basically the state at t = inf

    - If NR = FR we have problem.

      - assume higher power of solution

- sinusoidal

  - only look at forced response

  - V = Vm cos(w*theta + phi)

  - w = 2*pi*f (radians/sec)

  - phasors only show magnitude and phase

  - convert between sinusoid and phasor

  - XL = jwL

  - XC = 1/jCw

- sinusoidal power

  - p = vi = ½VmIm{cos q + cos(2wt + q)}

  - P=VmIm2cosθ=Vm2Im2cosθaverage =

- capacitor/inductor don’t use any power

- Effective value is RMS

  - V2=1Tv(t)2dt

    - for sine wave divide by root 2

Bode plots

- Get circuit in s-operator form

- use exponential eqiv circuit and replace s with jw when needed

- only concerned with the magnitude of H

- only asymptotic bode plots are required

- voltage gain = 20 log (Vout/vin)

- power gain = 10 log (Pout/Pin)

- bends down at pole

- bends up at zero

- low pass filter

- high pass filter

- H=sω01+sω0H=ωω01+ω2ω02

- poles form the exponents of the NR so the time constant can be determined by the poles

- Gain times bandwidth in an opamp is constant