Chapter 5, Problem 8

Obtain vo for each of these circuits

+

vo

-

1 mA

2 k

(a)

+

vo

-

2 V

1 V

10 k

2 k

(b)

Solution

(a)	Voltages at the input terminals are both zero

KCL gives		1 mA = (0 – vo)/2k

vo = -2 V

(b)	There is no voltage difference between the inverting and non-inverting inputs.

Also, no current can flow into the inverting input, and therefore no current flows through the 10 k resistance.

Therefore, we may apply KVL to give

1 – 2 = vo

ie	vo = -1 V

Chapter 5, Problem 10

Find the gain, vo/vs for this circuit

vs

20 k

10 k

10 k

+

vo

-

Solution

Since no current enters the OpAmp (and passes through the 20 kresistor), the voltage at the inverting input is vs

This is also the voltage at the non-inverting input. So

vs = vo x 10/(10 + 10)

vo/vs = 2

Chapter 5, Problem 15

Determine the ratio vo/is in the OpAmp circuit. Evaluate this ratio for R1 = 20 k, R2 = 25 k, R3 = 40 k

is

R1

R2

R3

+

vo

-

Solution

Voltage at the connection point for all three resistors is v and current into this point through R1 is

is = (0 – v)/R1 = v/R2 + (v – vo)/R3

Eliminate v to give

is(1 + R1/R2 + R1/R3) = -vo/R3

Therefore

vo/is = -(R1 + R3 + R1R3/R2)

Substituting the resistor values gives

vo/is = -(20 + 40 + 20*40/25)k = - 92 k

Chapter 5, Problem 19

Determine io in the following circuit

2 k4 k10 k

1 V    4 k

5 k

io

Solution

We convert the 1 V source and the 2 k resistance to a Norton equivalent with IN = 1/2k A and RN = 2 kW.

In parallel with the 4 k , this becomes 2 x 4/(2 + 4) = 4/3 k.

We then convert back to a Thevenin circuit with Vt = 4/3 k x 1/2k = 2/3 V

4/3 k4 k10 k

2/3 V

5 k

io

This is now a conventional inverting amplifier circuit, with a gain of -10/(4 + 4/3) = -15/8

Therefore		vo = -15/8 x 2/3 = -1.25 V

So		io = -1.25/5k – 1.25/10k

= -0.375 mA

Chapter 5, Problem 32

Calculate ix and vo in the following circuit. Find the power dissipated in the 60 k resistor.

4 mV

10 k

50 k

20 k

60 k

30 k

+

vo

-

ix

Solution

Let v = voltage at the output of the OpAmp. The circuit excluding the 20k, 30k and 60k resistors is a non-inverting amplifier, so

v = (1 + 50/10)(4 mV) = 24 mV

60║30 = 20 kW

Then, by voltage division,

vo = 20/(20 + 20) v = 12 mV

ix = v/(20 + 20)k

= 24 mV/40k

= 0.6 A

= 600 nA

Power in 60 k resistor is

P =  vo2/R

= 144*10-6/(60*103)

= 2.4 nW

Chapter 6, Problem 1

If the voltage across a 5 F capacitor is 2t e-3t V, find the current and power.

Solution

i = Cdv/dt = 5 * (2 e-3t – 6t e-3t)

= 10 (1 – 3t) e-3t A

p = vi

= 2t e-3t * 10 (1 – 3t) e-3t W

= 20 t(1 – 3t) e-6t W

Chapter 6, Problem 12

A voltage of 60 cos 4t V appears across the terminals of a 3 mF capacitor. Calculate the current through the capacitor and the energy stored in it from t = 0 to t = 0.125 s

Solution

i = C dv/dt = 3*10-3*60 (4)(-sin 4t)

= - 0.72  sin 4t A

p = vi = 60 (-0.72)  cos 4t sin 4t

= -21.6  sin 8t W

W=0tpdt=00.12521.6πsin8πtdt21.6π8πcos8π00.125=−5.4J

Chapter 6, Problem34

The current through a 10 mH inductor is 6 e-t/2 A. Find the voltage and power at t = 3 sec

Solution

v(t) = L di/dt = 10*10-3 * 6 * ½ e-t/2

= - 30 e-t/2 mV

v(3) = - 30 e-3/2

= -6.694 mV

p = vi = -180 e-t mW

p(3) = -180 e-3

= -8.962 mW

Chapter 6, Problem 43

The current in an 80 mH inductor increases from 0 to 60 mA. How much energy is stored in the inductor?

Solution

Stored energy = ½Li2

= ½*80*10-3*(60/1000)2

= 144*10-6 J

=144 J